Supplementary Table 1a. Rabbit B02, left RLN.

| Rabbit B02: left recurrent laryngeal nerve (sequential within-site paradigm) |  |  |  |
| --- | --- | --- | --- |
| Stimulation current | Scarred condition (Amplitude µV / Latency ms) | Unscarred condition (Amplitude µV / Latency ms) | Notes |
| 0.05 mA | no response | 2131 / 1.25 |  |
| 0.06 mA | no response | 2250 / 1.25 | Unscarred threshold ≤ 0.05 mA |
| 0.10 mA | no response | 2223 / 1.38 |  |
| 0.15 mA | no response | 2224 / 1.38 |  |
| 0.20 mA | no response | 2183 / 1.25 |  |
| 0.25 mA | 1158 / 1.63 | 2138 / 1.25 | Scarred threshold ≈ 0.25 mA |
| 0.30 mA | 1357 / 1.63 | 2232 / 1.38 |  |
| 0.35 mA | 733 / 1.75 | 2228 / 1.38 |  |
| 0.40 mA | 455 / 1.88 | 2228 / 1.38 |  |
| 0.45 mA | 652 / 1.88 | 2239 / 1.38 |  |
| 0.50 mA | 627 / 1.88 | 2206 / 1.38 |  |
| 0.60 mA | 953 / 1.75 | — | Unscarred not re-sampled |
| 0.70 mA | 1632 / 1.63 | — |  |
| 0.80 mA | 773 / 1.75 | — |  |
| 0.90 mA | 817 / 1.75 | — |  |
| 1.00 mA | 1559 / 1.50 | 2202 / 1.38 |  |
| 1.50 mA | 2001 / 1.50 | 2302 / 1.38 |  |
| 2.00 mA | 1303 / 1.63 | 2309 / 1.38 |  |

Dashes (—) indicate stimulation currents at which the unscarred condition was not re-sampled because the response had already plateaued at supra-threshold levels in the immediately preceding currents. This does not indicate absence of response. Latency values are rounded to the nearest 0.01 ms as acquired from the EMG instrument. Scar at the stimulation site was Grade 1 at re-exploration, consistent with animal B01.

Supplementary Table 1b. Rabbit B04, right RLN.

| Rabbit B04: right recurrent laryngeal nerve (sequential within-site paradigm) |  |  |  |
| --- | --- | --- | --- |
| Stimulation current | Scarred condition (Amplitude µV / Latency ms) | Unscarred condition (Amplitude µV / Latency ms) | Notes |
| 0.05 mA | — | 1461 / 1.25 | Unscarred threshold = 0.05 mA |
| 0.06 mA | — | 1494 / 1.38 |  |
| 0.07 mA | — | 1024 / 1.50 |  |
| 0.08 mA | — | 1207 / 1.50 |  |
| 0.09 mA | — | 1360 / 1.38 |  |
| 0.10 mA | 405 / 1.25 | 694 / 1.50 | Scarred threshold = 0.10 mA |
| 0.11 mA | 445 / 1.25 | 737 / 1.50 |  |
| 0.12 mA | 409 / 1.25 | 525 / 1.50 |  |
| 0.13 mA | 490 / 1.50 | 1133 / 1.38 |  |
| 0.14 mA | 432 / 1.50 | 1310 / 1.25 |  |
| 0.15 mA | 704 / 1.50 | 1340 / 1.25 |  |
| 0.20 mA | 567 / 1.50 | 1273 / 1.25 |  |
| 0.25 mA | 565 / 1.63 | 1242 / 1.25 |  |
| 0.30 mA | 444 / 1.63 | 1277 / 1.38 |  |
| 0.35 mA | 457 / 1.63 | 1275 / 1.25 |  |
| 0.40 mA | 380 / 1.50 | 1324 / 1.38 |  |
| 0.45 mA | 474 / 1.50 | 1080 / 1.38 |  |
| 0.50 mA | 469 / 1.63 | 1322 / 1.38 |  |
| 1.00 mA | 682 / 1.50 | 1186 / 1.38 |  |
| 1.50 mA | 628 / 1.50 | 1310 / 1.38 |  |
| 2.00 mA | 639 / 1.50 | 1330 / 1.38 |  |

Paradigm identical to Supplementary Table 1a. The absence of scarred-condition data below 0.10 mA reflects the scarred-site threshold. Absence of unscarred-condition data at 0.6-0.9 mA indicates currents not re-sampled after plateau was reached, not absence of response. Scar at the stimulation site was Grade 1 at re-exploration, consistent with animals B01 and B02.

Supplementary Table 1c. Cross-animal summary of scar-related electrophysiological changes.

| Parameter | B01 (Table 2, bilateral) | B02 left (Supplementary Table 1a) | B04 right (Supplementary Table 1b) | Direction of effect |
| --- | --- | --- | --- | --- |
| Stimulation threshold, unscarred | 0.09 mA | ≤0.05 mA | 0.05 mA | — |
| Stimulation threshold, scarred | 0.10 mA | ≈ 0.25 mA | 0.10 mA | Elevated at scarred sites |
| Amplitude at 0.5 mA, unscarred (µV) | ~1962 | 2206 | 1322 | — |
| Amplitude at 0.5 mA, scarred (µV) | 1891-2007 | 627 | 469 | Reduced at scarred sites (paradigm-dependent magnitude) |
| Scarred/unscarred amplitude ratio at 0.5 mA | ~99 % | 28 % | 35 % | Marked reduction with on-nerve interrogation through scar (B02, B04), near-unity with off-nerve probing beside minimal scar (B01) |
| Latency, unscarred (ms) | (see Table 2) | 1.25-1.38 | 1.25-1.38 | — |
| Latency, scarred (ms) | (see Table 2) | 1.50-1.88 | 1.25-1.63 | Prolonged (≈ 0-0.5 ms) |

Across all three Series 2 animals in which a scar-vs-unscarred electrophysiological comparison was performed (B01 via bilateral paradigm; B02 and B04 via sequential within-site paradigm), three directionally consistent findings were observed. Elevated stimulation threshold at scarred sites, reduced response amplitude at matched supra-threshold currents at scarred sites where the probe interrogated the nerve through scar (28-35% of the unscarred reference at 0.5 mA in B02/B04 under the sequential within-site paradigm), with B01 showing near-unity ratio because the probe was placed on adjacent unscarred tissue rather than through the scar (bilateral paradigm), and prolonged response latency at scarred sites (up to ~0.5 ms). The magnitude of effect differed between animals. In B01, peri-neural fibrosis was minimal at gross inspection, and the scarred-side threshold was elevated by only 0.01 mA relative to the contralateral unscarred side, consistent with the limited insulating effect of small-volume fibrosis when the probe was placed on adjacent unscarred tissue (bilateral paradigm). In B02 and B04, where the probe was placed on the nerve through the overlying scar (sequential within-site paradigm), thresholds were elevated approximately five-fold and two-fold, respectively, consistent with known inter-animal variability in peri-neural fibrosis density at re-exploration. These paradigm-independent, direction-concordant findings support the main conclusion presented in Table 2 and Section Effect of Peri-Neural Scar Tissue on Electrophysiological Parameters (the Results) of the manuscript.